ANNEX “A”

Technical Specifications

| - Background SLSU Main Campus (hereinafter, the Procuring Entity) intends to purchase various new technical and scientific equipment outlay for the College of Engineering and Technology to be able to improve its delivery of instructions by providing quality equipment to reinforce student learning by bridging theory or concepts to practice. |  |  |  |
| --- | --- | --- | --- |
| - Technical Specifications The goods must be brand-new and of a reputable brand. Refurbished equipment are not acceptable. The supplier shall provide SLSU Main Campus the Goods with the following minimum specifications: |  |  | Statementof Compliance |
| Item | Quantity | Minimum Specifications |  |
| 1 | 1 unit | - Bomb Calorimeter |  |
|  |  | TECHNICAL DATA: Measuring range max. [J] 40000 Measuring mode static jacket 22°C yes Measuring mode static jacket 30°C yes Measurements/h static jacket 4 Reproducibility static jacke (1g benzoic acid NBS39i) [%RSD] 0.15 Working temperature [°C] 20 - 30 Temperature measurement resolution [K] 0.0001 Cooling medium temperature [°C] 18 - 29 Cooling medium permissible operating pressure [bar] 1.5 Cooling medium tap water Type of cooling flow Flow rate [l/h] 50 - 60 Rec. flow rate at 18°C [l/h] 55 Oxygen operating pressure max. [bar] 40 Interface scale RS232 Interface printer RS232 Interface PC RS232 Oxygen filling yes Degasification yes Decomposition vessel integrated yes Analysis according to DIN 51900 yes Analysis according to ISO 1928 yes Dimensions (W x H x D) [mm] 290 x 280 x 300 Weight [kg] 15 Permissible ambient temperature [°C] 5 - 40 Permissible relative humidity [%] 80 Protection class according to DIN EN 60529 IP 20 RS 232 interface yes USB interface yes Voltage [V] 100 - 240 Frequency [Hz] 50/60 Power input [W] 120 PACKAGED WITH: a. Set of Spare Parts, approximately for 1000 Experiments, C1.101. b. Ignition Wire, Standard Kantal C1.103 c. Benzoic Acid, Blister 50 pcs. d. Combustion Chamber e. Industrial Oxygen Gas Tank (Standard Size of the Tank 5-feet) equipped with Pressure Guage / Regulator f. Software g. Notebook Computer pre-installed with Software |  |
| 2 | 1 unit | - Furnace 24L |  |
|  |  | TECHNICAL SPECIFICATIONS: Model: Lift Door, LT24/11 Capacity: 24 Liters Tmax: 1100ºC Inner Dimensions (mm): 280W x 340D x 250H Outer Dimensions (mm): 560W x 660D x 650H Connected Load (kg): 4.5 Electrical Connection: 3-phase Weight in Kg.: 75 Minnutes to Tmax: 95 INCLUSION DURING DELIVERY: a. AVR, 10 KVA b. Steel Collecting Pan, Tmax 1100ºC c. Tongs |  |
| 3 | 1 unit | - Moisture Analizer |  |
|  |  | TECHNICAL SPECIFICATIONS: Max. weighing capacity 70 g Repeatability, typical Starting at an initial sample weight of approx. 1 g: ± 0.2% for initial sample weight approx. > 5 g: ± 0.05% Readability 1 mg, 0.01% Typical sample quantity 5 – 15 g Reading display Moisture content in % M and % g ¦ dry matter in % S and g ¦ ATRO in % M/S Temperature range and settings: 40°C–160°C, in increments of 1 degree Standby temperature selectable from 40 to 100°C Sample heating Infrared heating using a metal tube heater Heating programs Standard drying, gentle drying Shutoff parameter Fully automatic, semi-automatic, manual and with timer settings Access to sample chamber Removable hood with wide opening angle, SoftClose mechanism Measuring program : 1 program saved in a non-volatile memory (freely selectable method parameters) Memory for data storage Results are saved until the start of the next measurement Sample inspection LED-illuminated sample chamber, inspection window with grid above the hood Interface Mini USB, automatic printer detection, direct transfer to Microsoft® programs |  |
| 4 | 1 unit | - Precision Balance 3200g 0.01g |  |
|  |  | TECHNICAL SPECIFICATIONS: Capacity (g) : 3200 Display : Backlit LCD Readability (g) : Linearity (g) : 0.03 Repeatability (g): 0.01 Weighing units : g, mg, kg, ct, oz, troy, lb, pennyweight, grain, Newton, momme mesghal, tael (3), tical, tola, baht Stabilization time: 1.5 Second Power (VAC) : 120/220 Interface :RS-232 Languages : Multi Language |  |
| 5 | 1 unit | - Analytical Balance 220g, 0.0001g |  |
|  |  | TECHNICAL SPECIFICATIONS: Weighing Capacity: 220g Readability : 0.0001g or 0.1mg Repeatability : 0.1mg Linearity : 0.2mg Typical Stabilization Time: 2.5S Weighing Pan Size (mm): Ø90 Dimensions D x W x H (mm): 230 x 303 x 330 Calibration and Adjustment: External |  |
| 6 | 1 unit | - Hot Plate |  |
|  |  | TECHNICAL DATA: Number of stirring positions 1 Stirring quantity max. per stirring position (H2O) [l] 10 Motor rating output [W] 1.5 Direction of rotation left Speed control scale 0 - 6 Speed range [rpm] 100 - 1500 Stirring bar length max. [mm] 80 Heat output [W] 1000 Heating temperature range [°C] 50 - 500 Heat control LED Heating rate heating plate [K/min] 5 Connection for ext. temperature sensor ETS-D5 Fixed safety circuit [°C] 550 Set-up plate material ceramic Set-up plate dimensions [mm] 180 x 180 Dimensions (W x H x D) [mm] 220 x 105 x 330 Weight [kg] 5 Permissible ambient temperature [°C] 5 - 40 Permissible relative humidity [%] 80 Protection class according to DIN EN 60529 IP 21 Voltage [V] 230 / 120 / 100 Frequency [Hz] 50/60 Power input [W] 1020 |  |
| 7 | 1 unit | - UPS 6KVA/5.4KW |  |
|  |  | Memo Plus S 6KVA 6KVA/5.4W 0.9PF with Internal Battery (1:1) Input Voltage 220/230/240VAC Output Voltage 220/230/240VAC |  |
| 8 | 1 unit | - Air Conditioner 2.5HP Split D Smart Inverter |  |
|  |  | Cooling Capacity : 2.5 HP Power Consumption : 1740W Energy Efficient Ratio : 11.77 kJ/Wh Dimensions (HxWxD) : 295mm x 990mm x 263mm Refrigerant : R32 |  |
| 9 | 1 unit | - Total Station |  |
|  |  | Specifications Range with Nikon specified prisms - Good conditions (No haze, visibility over 40 km (25 miles)) With reflector sheet (5 × 5 cm): 1.5 m to 200 m (4.9 ft to 787 ft) With single prism 6.25 cm (2.5 in): 3,000 m (9,840 ft) ■ Range reflectorless mode - KGC (18%)1 Good: 250 m (820 ft) Normal: 200 m (656 ft) Difficult: 150 m (492 ft) Accuracy ISO 17123-4 (Precise mode) - Prism2 : ±(2+2 ppm × D) mm - Reflectorless/Reflector sheet3 ±(3+3 ppm × D) mm ■ Measuring interval4 - Prism mode Precise mode: 1.1 sec. Normal mode: 0.8 sec. - Reflectorless mode Precise mode: 1.2 sec. Normal mode: 1.0 sec. - Least count Precise mode: 1 mm (0.002 ft) Normal mode: 10 mm (0.02 ft) Angle Measurement ■ ISO 17123-3 accuracy (horizontal and vertical): 2"/0.6 mgon, 5"/1.5 mgon ■ Reading system: Photoelectric detection by incremental encoder ■ Circle diameter: 88 mm (3.46 in) ■ Horizontal angle: 2":Diametrical, 5":Single ■ Vertical angle: Single ■ Minimum increment (Degree, Gon, MIL6400): Degree: 1/5/10", Gon: 0.2/1/2 mgon, MIL6400: 0.005/0.02/0.05 mil Telescope ■ Tube length: 125 mm (4.9 in) ■ Image: Erect ■ Magnification: 30× (18x/36x with optional eyepieces) ■ Effective diameter of objective: 45 mm (1.77in) - EDM: 50 mm (1.97 in) ■ Field of view: 1°20' ■ Resolving power: 3" ■ Minimum focusing distance: 1.5 m (4.9 ft) ■ Laser Pointer: Coaxial Red Light Tilt Sensor ■ Type: Dual axis ■ Method: Liquid-electric detection ■ Compensation range: ±3' Communications ■ Communication ports: 1 x serial (RS-232C) Wireless Communications ■ Integrated Bluetooth General Specifications ■ Level vials - Sensitivity of Plate level vial: 30"/2 mm - Sensitivity of Circular level vial: 10'/2 mm ■ Optical plummet - Image: Erect - Magnification: 3× - Field of view: 5° - Focusing range: 0.5 m (1.6 ft) to ∞ ■ Display: 2":Both sides, backlit, graphic LCD (128 × 64 pixel), 5":Single side, backlit, graphic LCD (128 × 64 pixel) ■ Point memory: 50,000 records COMPLETE WITH: 1 MAIN UNIT 1 CARRYING CASE 1 BATTERY 1 CHARGER 1 PRISM POLE, TELESCOPIC TYPE WITH READINGS, SINGAPORE 1 PRISM WITH HOLDER AND TARGET 1 ALUMINUM HEAVY DUTY DUAL LOCK TRIPOD, SINGAPORE 1 CALIBRATION CERTIFICATE GOOD FOR 1 YEAR |  |
| 10 | 1 unit | - Sieve Shaker |  |
|  |  | Inclusions: Main unit 1 unit clamping device 1 pc bottom collecting pan Features: digital display of performance and time suitable for dry and wet sieving excellent separation efficiency even with short sieving times efficient electromagnetic drive 3-D throwing motion which ensures optimum use of the open sieve area and lets the sample move equally over the whole sieving surface sieve stack up to 510 mm height free adjustment of all process parameters (time, performance) optional comfort clamping device optional evaluation software maintenance-free easy operation, ergonomic design |  |
| 11 | 1 unit | - Manual Base Unit w/Built-in Power Supply |  |
|  |  | Voltage 100-250 V ac Current 0.4-0.65 A Frequency 50/60 Hz Physical Characteristics Intended Location On a table able to support the weight of the equipment Dimensions (H x W x D) 152 x 305 x 356 mm (6 x 12 x 14 in) Net Weight 3.1 kg (6.9 lb) |  |
| 12 | 1 unit | - DeLux PICmicro Microcontroller starter Pack |  |
|  |  | Features & Benefits Ideal for learning about e-system development at a wide range of academic levels Includes mounting platform, screws and bolts Wide variety of optional parts to enhance learning Fullfills a large range of learning objectives and levels Flexible, portable, and expandable Saves time and money for the school Well supported and documented Supplied in rugged plastic trays for convenient storage and protection |  |
| 13 | 1 unit | - Digital Communications 1 |  |
|  |  | Features & Benefits Each circuit block contains a modulator for transmission and a demodulator for reception. Built-in channel simulator and speaker amp circuitry The channel simulator circuit block enables students to investigate the effects of noise and channel bandwidth on pulse and digital modulation signals The speaker amp circuit block permits students to connect a speaker and listen to the signals. Communication signals are synchronized for easy display Concepts of Digital Communications, Circuit Board Familiarization PAM Signal Generation, Demodulation, PAM TDM Transmission and Reception PTM Signal Demodulation and Generation PCM Signal Generation and Demodulation, Signal Time-Division Multiplexing DM Transmitter, Receiver and Noise Channel Bandwidth and Noise Troubleshooting Basics and Troubleshooting Digital Communications 2 Circuits |  |
| 14 | 1 unit | - Digital Signal Processor |  |
|  |  | The module and courseware provide an introduction to DSPs, some typical DSP applications, and how DSPs are programmed. Programming experience may be helpful but is not required to perform the exercises. The module features the TMS320C5535 Fixed-Point Digital Signal Processor from Texas Instruments. Built-in accessories include a DC source, microphone pre-amplifier, audio amplifier and speaker. Built-in peripherals include a CODEC for A/D and D/A conversion, an I/O interface with DIP switches and an LCD display, two interrupt pushbuttons, two USB ports, and an auxiliary I/O block. Familiarization with DSPs and DSP programming, Overview of the DSP Circuit Board, The Integrated Development Environment (IDE) and Project Structure DSP Architecture, Processor Arithmetic, The Data Computation Unit, Memory, Addressing I/O and Peripherals, An Application Using I/Os and Peripherals DSP Real-time Processing, Sampling and Analog-to-Digital/Digital-to-Analog Conversion, The Fast Fourier transform (FFT), Optimizing DSP applications Signal Processing Applications, FIR and IIR Filters |  |
| 15 | 1 unit | - Digital Logic Fundamentals |  |
|  |  | The circuit board contains the following circuits: Clock Input Signals Open Collector Tri-State Output AND/NAND Set/Reset Flip-Flop TTL/CMOS Comparison OR/NOR D-type Flip-Flop Data Bus Control JK Flip-Flop and XOR/XNOR circuit blocks for students to understand and troubleshoot +5 V regulated supply |  |
| 16 | 1 unit | - DC Fundamentals |  |
|  |  | A dc milliammeter is provided with the circuit board. The circuits found on this board include: Batteries Switches Ohm’s Law Series Circuit Parallel Circuit Series-Parallel Circuit Power Linear/Non-Linear Variable Resistor Voltage Divider Voltmeter/Ammeter/Ohmmeter |  |
| 17 | 1 unit | - AC 1 Fundamentals |  |
|  |  | Generator Impedance AC/DC Waveforms Phase Angle Inductance/Inductive Reactance Transformer Capacitance/Capacitive Reactance RC Time Constants RC/RL Wave Shapes |  |
| 18 | 3 units | - Wireless Smart Dynamics Cart |  |
|  |  | Specifications Force Force Range: ± 100 N Position Resolution: 0.1 N Accuracy : ± 1% Maximum sampling rate: 500 samples/second normally 5000 samples/second in burst mode Position Resolution: ± 0.2 mm Velocity Max velocity: ± 3 m/s Max sample rate : 100 samples/second Acceleration Range: ± 16g (g= 9.8 m/s2) Max sample rate: 500 samples/second Bluetooth distance Max wireless range: 30 m (unobstructed) |  |
| 19 | 3 units | - Centripetal Force Apparatus w/ Photogate |  |
|  |  | Includes: Rectangular Frame Support Legs (2) Rotating Assembly consisting of Rotational Shaft, Encoder Wheel, 3-Step Pulley, Rotating Beam, Bearing Housing Sliding Carriage to hold the test masses Counterbalance Carriage to balance the rotating beam Swivel Assembly (connects Sliding Carriage to a Dual-Range Force Sensor) Ultra Pulley Ultra Pulley Bracket (preinstalled on Assembly) Photogate Bracket with 1/4 x 20 mounting screw (preinstalled on Assembly) Dual-Range Force Sensor Bracket (preinstalled on Assembly) 50 g masses (4) 100 g masses (4) Hook (attaches a Wireless Dynamic Sensor System to the Sliding Carriage) WDSS Mounting Screw and T Nut to secure a WDSS Photogate Specifications Power req: 5 VDC at 40 mA Infrared source: Peak at 880 nm Dimension: 75 mm gate width Output is high and LED off for unblocked gate Output is low and LED on for blocked gate Beam approximately 9 mm from the end of the gate arms |  |
| 20 | 3 units | - Basic Optics Light Source |  |
|  |  | Specifications Bulb type: G4 halogen, 12V, 10W Input Power (to included AC Adapter): 100 to 240 VAC, 50 to 60 Hz Dimensions: 16 cm x 10 cm x 9 cm |  |
| 21 | 1 unit | - ProScope Microscope Camera 5mp WiFi |  |
|  |  | Specifications Resolution: 5 megapixel Sensor type: CMOS Magnification: 10x Eyepiece tube compatibility: 23, 30, and 30.5 mm (with included adapters) Wi-Fi compatibility: iOS, Android (supports up to 7 device simultaneously) USB compatibility: Windows, Mac, Chromebook USB cable length: 1.5 m Power source: Internal rechargeable battery or USB Battery life: 8 hrs |  |
| 22 | 1 unit | - Dual Extruder 3D Printer |  |
|  |  | Build volume : 230 x 145 x 145mm Resolution: up to 100 microns, 200 microns, 300 microns Speed: Min. 40- Max. 200mm/s Flow rate: 24cc/hr Precision: XY 0.011mm Z 0.0025mm Nozzle: 0.4mm diameter Dual Extruder Pro II nozzles Equipment size: 480 x 400 x 335 Package size: 565 x 430 x 535 Print surface: Heated Bed Layer resolution: 0.1mm – 0.5mm Print Technology: FDM Weight: 25kgs Software: Flashprint File types: STL/OBJ OS Requirement: Windows/Mac/ Linux Acrylic + Aluminum casing Material: ABS, non-toxic PLA only Open-source filament system Connectivity: SD Card support, USB printing direct Electrical: 100-240V AC input 300W |  |
| 23 | 1 unit | - PRO Tablet 12-inch w/Pen |  |
|  |  | Stylus Pen LED-backlit IPS LCD, capacitive touchscreen, 16M colors 12.9 inches, 515.3 cm2 (~76.4% screen-to-body ratio) 2732 x 2048 pixels, 4:3 ratio (~265 ppi density) Scratch-resistant glass, oleophobic coating True-tone display 64GB, 4GB RAM 12 MP (f/1.8, 1/3"), phase detection autofocus, OIS, quad-LED dual-tone flash Geo-tagging, simultaneous 4K video and 8MP image recording, touch focus, face/smile detection, HDR 2160p@30fps, 1080p@30/60fps, 1080p@120fps, 720p@240fps 7 MP (f/2.2, 32mm), 1080p@30fps, 720p@240fps,, face detection, HDR, panorama A-GPS, GLONASS (Wi-Fi + Cellular model only) Non-removable Li-Ion 10,891 mAh battery (41 Wh) 305.7 x 220.6 x 6.9 mm (12.04 x 8.69 x 0.27 in) 677 g (Wi-Fi) / 692 g (LTE) (1.49 lb) |  |
| - Terms of Technical Support |  |  |  |
| 1 | The Supplier shall provide 24/7 telephone, e-mail, chat, and remote technical support services covering all Goods with a two-day response time from SLSUs posting of support request. |  |  |
| 2 | The Supplier shall provide next-business-day onsite support for technical issues not resolved through the actions described in the preceding paragraph. Hardware defects/errors in any component of the Goods shall automatically require onsite support. |  |  |
| - Warranty |  |  |  |
| 1 | The Supplier shall post a warranty security for a minimum period of one (1) year after acceptance by SLSU of the delivered items. The obligation for the warranty shall be covered by either retention money in an amount equivalent to at least one percent (1%) of every progress payment, or special bank guarantee equivalent to at least one percent (1%) of the total contract price. The said amounts shall only be released after the lapse of the warranty period and all the conditions imposed under the contract have been fully met. |  |  |
| 2 | The Supplier shall replace Goods found to have satisfactory defects immediately inclusive of cost of replacement/repair of parts and labor. |  |  |
| - Delivery and Retrieval |  |  |  |
| 1 | The Supplier shall deliver the goods within thirty (30) working days from the receipt of the Notice to Proceed. |  |  |
| 2 | The Supplier must repair Goods that are under warranty within five (5) working days. |  |  |
|  | In case replacement or repair will take longer than five (5) working days, the Supplier shall provide a temporary replacement unit of equal capability and features as the item to be replaces/repaired. |  |  |
| - Terms of Payment |  |  |  |
| 1 | All payments shall be inclusive of Value-Added Tax (VAT) and other lawful charges. |  |  |
| - Miscellaneous |  |  |  |
| 1 | The Supplier shall agree with the definitions and terms found in the General Conditions of Contract (GCC) and Special Conditions of Contract (SCC) contained in the Philippine Bidding Documents for Procurement of Goods issued by the Government Procurement Policy Board (GPPB). |  |  |

Note:

Bidder must state compliance to each of the provisions in the Terms of Reference/Technical Specifications, as well as to the Schedule of Requirements.  The STATEMENT OF COMPLIANCE must be signed by the authorized representative of the Bidder, with proof of authority to sign and submit the bid for and in behalf of the Bidder concerned.  If the Bidder is a joint venture, the representative must have authority to sign for and in behalf of the partners to the joint venture.

Conformé:

_______________________________________

[Signature/s]

[Name of Bidder’s Authorized Representative/s]

_______________________________________

[Position]

_________________________

[Date]